CO3 - ASSESSMENT TOOL 2

SIMULATION TASK

PROBLEM –

5Gmm Wave Beamforming Decimator

Simulate a decimation stage in a 5Gmm Wave receiver and evaluate bandwidth preservation after sample rate reduction.

Given Data:

● Sampling Frequency = 245.76 MHz

● Decimation Factor = 8

Tasks:

1. Design an anti-alias filter.

2. Implement decimation.

3. Analyse output spectrum.

4. Verify bandwidth preservation.

5. Measure computational savings.

6. Discuss practical significance.

PROGRAM -

clc;

clear;

close all;

scriptPath = fileparts(mfilename('fullpath'));

if isempty(scriptPath)

scriptPath = pwd;

end

outputFolder = fullfile(scriptPath,'Decimation_Output');

if ~exist(outputFolder,'dir')

mkdir(outputFolder);

end

Fs = 245.76e6;

M = 8;

Fs_new = Fs/M;

T = 1/Fs;

t = 0:T:50e-6;

f1 = 5e6;

f2 = 10e6;

x = sin(2*pi*f1*t) + 0.7*sin(2*pi*f2*t);

Fc = 14e6;

N = 80;

b = fir1(N,Fc/(Fs/2),'low');

x_filtered = filter(b,1,x);

y = decimate(x,M,'fir');

t_new = (0:length(y)-1)/Fs_new;

Nfft = 4096;

X = fftshift(abs(fft(x,Nfft)));

f = linspace(-Fs/2,Fs/2,Nfft);

Y = fftshift(abs(fft(y,Nfft)));

f2_axis = linspace(-Fs_new/2,Fs_new/2,Nfft);

original_samples = length(x);

new_samples = length(y);

saving = (1-new_samples/original_samples)*100;

disp('--------------------------------')

disp(['Original Sampling Frequency = ',num2str(Fs/1e6),' MHz'])

disp(['New Sampling Frequency = ',num2str(Fs_new/1e6),' MHz'])

disp(['Original Samples = ',num2str(original_samples)])

disp(['Decimated Samples = ',num2str(new_samples)])

disp(['Computational Saving = ',num2str(saving),' %'])

disp('--------------------------------')

fig1 = figure;

subplot(2,1,1)

plot(t*1e6,x)

grid on

title('Original Signal')

xlabel('Time (\mus)')

ylabel('Amplitude')

subplot(2,1,2)

plot(t_new*1e6,y)

grid on

title('Decimated Signal')

xlabel('Time (\mus)')

ylabel('Amplitude')

exportgraphics(fig1,...

fullfile(outputFolder,'Time_Domain_Comparison.png'),...

'Resolution',300);

fig2 = figure;

subplot(2,1,1)

plot(f/1e6,X)

grid on

title('Spectrum Before Decimation')

xlabel('Frequency (MHz)')

ylabel('Magnitude')

subplot(2,1,2)

plot(f2_axis/1e6,Y)

grid on

title('Spectrum After Decimation')

xlabel('Frequency (MHz)')

ylabel('Magnitude')

exportgraphics(fig2,...

fullfile(outputFolder,'Spectrum_Comparison.png'),...

'Resolution',300);

fig3 = figure;

freqz(b,1,1024,Fs)

title('Anti-Alias FIR Filter Response')

exportgraphics(fig3,...

fullfile(outputFolder,'AntiAlias_Filter_Response.png'),...

'Resolution',300);

disp(' ')

disp('Program executed successfully.')

disp(['Images saved in: ',outputFolder])

disp('Saved Files:')

disp('1. Time_Domain_Comparison.png')

disp('2. Spectrum_Comparison.png')

disp('3. AntiAlias_Filter_Response.png')

OUTPUT –

FIG 1 - Anti Alias Filter Response

FIG 2 – Spectrum Comparison

FIG 3 – Time Domain Comparison